INTRODUCTION:

This activity is used to write a MIPS assembly program to perform the arithmetic computation for a given C++ pseudo code and calculate the factorial of a given number. After writing the MIPS program, it has to be executed step by step to record the values of Machine code, registers and memory.

MY GROUP:

Name: Student_Team 6

Harish Marepalli – 016707314

Tirumala Saiteja Goruganthu – 016707210

SOURCE CODE (Task 1):

Task1: The total number of lines are 26

ori $a0, $0, 0x8000		#line 1: store the value 0x8000 in a variable a

ori $a1, $0, 0x00A9		#line 2: store the value 0x00A9 in a variable b

ori $s0, $0, 1974		#line 3: store the value 1974 in a variable c

multu $a0, $a0			#line 4: multiply a with a

mflo $s1	#line 5: move li content to x. Store in s1 register (no overflow condition)

addi $t0, $0, 0x20		#line 6: store the base address in a temporary register t0

sw $s1, 0($t0)			#line 7: store x in a location 0x20

multu $s1, $a1			#line 8: multiply x with b

mfhi $s2			#line 9: move hi content to y

sw $s1, 4($t0)			#line 10: store lo content in a location 0x24[y]

sw $s2, 8($t0)			#line 11: store hi content in a location 0x28[y+4]

srl $s2, $s1, 16			#line 12: right shift y.lo value

jal compute	#line 13: jump to compute label to calculate the given formula

sw $s0, 12($t0)			#line 14: store c value in a location 0x2c

addi $t3, $0, 1	#line 15: store the value '1' in t2 register for future beq comparison

while: 		slti $t1, $s0, 1665		#line 16: check if c<1665 => $t1=1 else $t1=0

beq $t1, $t3, done	#line 17: branch to done if $t1==$t3 since we should come out of the while loop

jal compute	#line 18: jump to compute label to calculate the given formula

j while				#line 19: loop back to while

compute:           divu $s2, $s0	#line 20: divide y with c. The quotient gets stored in lo and the remainder gets stored in hi

mflo $t1			#line 21: move the quotient of previous result into t1 register

add $t1, $s0, $t1	#line 22: add c to the previous result and store the result in a temporary t1 register

srl $s0, $t1, 1			#line 23: right shift by '1' to essentially divide by 2

jr $ra				#line 24: jump to return address given by ra register

done:                 sll $s0, $s0, 8	#line 25: logic left shift c value by 8 and store it back in c ($s0)

sw $s0, 16($t0)			#line 26: store c value in a location 0x30

CMPE200 Assignment 3 Task 1 Test Log

Algorithm 1

Programmer’s Name:  Harish Marepalli

Date: 09/24/2022

| Adr | MIPS Instruction | Machine Code | Registers |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | $a0 | $a1 | $s0 | $s1 | $s2 |
| 3000 | ori $a0, $0, 0x8000 | 0x34048000 | 0x00008000 | 0x00000000 | 0x00000000 | 0x00000000 | 0x00000000 |
| 3004 | ori $a1, $0, 0x00A9 | 0x340500a9 | 0x00008000 | 0x000000a9 | 0x00000000 | 0x00000000 | 0x00000000 |
| 3008 | ori $s0, $0, 1974 | 0x341007b6 | 0x00008000 | 0x000000a9 | 0x000007b6 | 0x00000000 | 0x00000000 |
| 300c | multu $a0, $a0 | 0x00840019 | 0x00008000 | 0x000000a9 | 0x000007b6 | 0x00000000 | 0x00000000 |
| 3010 | mflo $s1 | 0x00008812 | 0x00008000 | 0x000000a9 | 0x000007b6 | 0x40000000 | 0x00000000 |
| 3014 | addi $t0, $0, 0x20 | 0x20080020 | 0x00008000 | 0x000000a9 | 0x000007b6 | 0x40000000 | 0x00000000 |
| 3018 | sw $s1, 0($t0) | 0xad110000 | 0x00008000 | 0x000000a9 | 0x000007b6 | 0x40000000 | 0x00000000 |
| 301c | multu $s1, $a1 | 0x02250019 | 0x00008000 | 0x000000a9 | 0x000007b6 | 0x40000000 | 0x00000000 |
| 3020 | mfhi $s2 | 0x00009010 | 0x00008000 | 0x000000a9 | 0x000007b6 | 0x40000000 | 0x0000002a |
| 3024 | sw $s1, 4($t0) | 0xad110004 | 0x00008000 | 0x000000a9 | 0x000007b6 | 0x40000000 | 0x0000002a |
| 3028 | sw $s2, 8($t0) | 0xad120008 | 0x00008000 | 0x000000a9 | 0x000007b6 | 0x40000000 | 0x0000002a |
| 302c | srl $s2, $s1, 16 | 0x00119402 | 0x00008000 | 0x000000a9 | 0x000007b6 | 0x40000000 | 0x00004000 |
| 3030 | jal compute | 0x0c000c13 | 0x00008000 | 0x000000a9 | 0x000007b6 | 0x40000000 | 0x00004000 |
| 3034 | sw $s0, 12($t0) | 0xad10000c | 0x00008000 | 0x000000a9 | 0x000003df | 0x40000000 | 0x00004000 |
| 3038 | addi $t3, $0, 1 | 0x200b0001 | 0x00008000 | 0x000000a9 | 0x000003df | 0x40000000 | 0x00004000 |
| 303c | while: slti $t1, $s0, 1665 | 0x2a090681 | 0x00008000 | 0x000000a9 | 0x000003df | 0x40000000 | 0x00004000 |
| 3040 | beq $t1, $t3, done | 0x112b0007 | 0x00008000 | 0x000000a9 | 0x000003df | 0x40000000 | 0x00004000 |
| 3044 | jal compute | 0x0c000c13 |  |  |  |  |  |
| 3048 | j while | 0x08000c0f |  |  |  |  |  |
| 304c | compute: divu $s2, $s0 | 0x0250001b | 0x00008000 | 0x000000a9 | 0x000007b6 | 0x40000000 | 0x00004000 |
| 3050 | mflo $t1 | 0x00004812 | 0x00008000 | 0x000000a9 | 0x000007b6 | 0x40000000 | 0x00004000 |
| 3054 | add $t1, $s0, $t1 | 0x02094820 | 0x00008000 | 0x000000a9 | 0x000007b6 | 0x40000000 | 0x00004000 |
| 3058 | srl $s0, $t1, 1 | 0x00098042 | 0x00008000 | 0x000000a9 | 0x000003df | 0x40000000 | 0x00004000 |
| 305C | jr $ra | 0x03e00008 | 0x00008000 | 0x000000a9 | 0x000003df | 0x40000000 | 0x00004000 |
| 3060 | done: sll $s0, $s0, 8 | 0x00108200 | 0x00008000 | 0x000000a9 | 0x0003df00 | 0x40000000 | 0x00004000 |
| 3064 | sw $s0, 16($t0) | 0xad100010 | 0x00008000 | 0x000000a9 | 0x0003df00 | 0x40000000 | 0x00004000 |
| 3068 |  |  |  |  |  |  |  |
| 306C |  |  |  |  |  |  |  |

| Memory contents |  |  |  |  |
| --- | --- | --- | --- | --- |
| Word @ 0x20 | Word @ 0x24 | Word @ 0x28 | Word @ 0x2C | Word @ 0x30 |
| 0x40000000 | 0x40000000 | 0x0000002a | 0x000003df | 0x0003df00 |

SCREEN CAPTURES:

- Screenshot of the Task1 code in the MARS editor.

- Screenshot of the execution window after assembling and before executing the code in MARS.

- Screenshot of the execution window after executing the code in MARS.

SOURCE CODE (Task 2):

Task2: Total number of lines are 10

ori $a0, $0, 5		#line 1: store the value 5 in a variable n($a0)

sw $a0, 0($t0)	#line 2: store the value of n in memory location at address 0x00. $t0 by default is 0x00

ori $t1, $0, 1		#line 3: store the value 1 in a variable f($t1)

while: 	beq $a0, $0, done	#line 4: branch to done if n = 0 as we have to come out of the while condition

mult $t1, $a0		#line 5: multiply f with n

mflo $t1		#line 6: move the content from lo to f

addi $a0, $a0, -1	#line 7: decrement the value of n by adding it with -1

j while			#line 8: loop back to while

done: 	sw $t1, 16($t0)		#line 9: store the value of f in memory location at address 0x10

lw $s0, 16($t0)		#line 10: load the value at address 0x10 to the register s0

CMPE200 Assignment 3 Task 2 Test Log

Algorithm 2

Programmer’s Name:  Harish Marepalli

Date: 09/24/2022

| Adr | MIPS Instruction | Machine Code | Registers |  |  |  | Memory Content |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | $a0 | $s0 | $t0 | $t1 | Word @ 0x00 | Word @ 0x10 |
| 3000 | ori $a0, $0, 5 | 0x34040005 | 0x00000005 | 0x00000000 | 0x00000000 | 0x00000000 | 0x00000000 | 0x00000000 |
| 3004 | sw $a0, 0($t0) | 0xad040000 | 0x00000005 | 0x00000000 | 0x00000000 | 0x00000000 | 0x00000005 | 0x00000000 |
| 3008 | ori $t1, $0, 1 | 0x34090001 | 0x00000005 | 0x00000000 | 0x0000000 | 0x00000001 | 0x00000005 | 0x00000000 |
| 300c | while: beq $a0, $0, done | 0x10800004 | 0x00000000 | 0x00000000 | 0x00000000 | 0x00000078 | 0x00000005 | 0x00000000 |
| 3010 | mult $t1, $a0 | 0x01240018 | 0x00000001 | 0x00000000 | 0x00000000 | 0x00000078 | 0x00000005 | 0x00000000 |
| 3014 | mflo $t1 | 0x00004812 | 0x00000001 | 0x00000000 | 0x00000000 | 0x00000078 | 0x00000005 | 0x00000000 |
| 3018 | addi $a0, $a0, -1 | 0x2084ffff | 0x00000000 | 0x00000000 | 0x00000000 | 0x00000078 | 0x00000005 | 0x00000000 |
| 301c | j while | 0x08000c03 | 0x00000000 | 0x00000000 | 0x00000000 | 0x00000078 | 0x00000005 | 0x00000000 |
| 3020 | done: sw $t1, 16($t0) | 0xad090010 | 0x00000000 | 0x00000000 | 0x00000000 | 0x00000078 | 0x00000005 | 0x00000078 |
| 3024 | lw $s0, 16($t0) | 0x8d100010 | 0x00000000 | 0x00000078 | 0x00000000 | 0x00000078 | 0x00000005 | 0x00000078 |
| 3028 |  |  |  |  |  |  |  |  |

SCREEN CAPTURES:

- Screenshot of the Task2 (Factorial) code in the MARS editor.

- Screenshot of the execution window after assembling and before executing the code in MARS.

- Screenshot of the execution window after executing the code in MARS.

DISCUSSION SECTION

The explanation of a few instructions in the sample code with the help of the MIPS reference data card.

- multu rs, rt => rs*rt

Perform multiplication of the contents present in the two registers. A part of the result is stored in lo register and the other part is stored in hi register if the result is greater than 32 bits.

- divu rs, rt => rs/rt

Perform division of the contents present in the two registers. The quotient is stored in lo register and the remainder is stored in ho register.

- mflo rd => rd = lo

Moves the content from lo register to the rd register.

- mfhi rd => rd = hi

Moves the content from hi register to the rd register.

- sll rd, rt, sh => rd = rt<<sh

It left shifts the content in the register rt by sh times.

- srl rd, rt, sh => rd = rt>>sh

It right shifts the content in the register rt by sh times.

- j addr => PC = addr

Jumps to the specified address by placing the value in PC.

- jal addr => $ra = next PC value, PC = addr

Jump and link to the given address by putting its value in the PC as well as storing the next PC in $ra.

- jr $ra => PC = ra

After the completion of a subroutine, PC must go back to the next instruction after the caller method. jr is used for this.

Following are the observations related to some of the machine codes:

- Explanation with respect to the Task1 code 19th line (j while)

i.e., j while

0x0000303c	while: slti $t1, $s0, 1665

…

0x00003048	j while

Machine code of it is written below:

It is a j-type instruction

6 bits		                             26 bits

| Opcode | Target |
| --- | --- |

32 bits

Opcode for j instruction is 000010

The address is 32 bit, but only 26 bits are available. To solve this problem, ‘Jump Target Addressing’ method is followed.

Target address in hexadecimal => 0x0000303c

Target address in binary format => 0000 0000 0000 0000 0011 0000 0011 1100

Here, remove the first 4 bits and last 2 bits that are added from next PC and the shift left operation respectively to get the required 26 bits immediate.

Immediate => 0000 0000 0000 0011 0000 0011 11

Finally, the machine code comes as opcode + immediate

i.e., 000010 0000 0000 0000 0011 0000 0011 11

=> 0000 1000 0000 0000 0000 1100 0000 1111

So, the machine code in hexa decimal format is 0x08000c0f

- Explanation with respect to the Task1 code 17th line (beq)

i.e., beq $t1, $t3, done

0x00003040	beq $t1, $t3, done

…

0x00003060	done

Machine code of it is written below:

It is a I-type instruction

6 bits		       5 bits	       5 bits		  	    16 bits

| Opcode | Rs | Rt | Immediate |
| --- | --- | --- | --- |

32 bits

The machine code for the above beq instruction can be found using the 16-bit immediate, which will be positive value since the branching happens downwards direction.

It is known that the branch target addressing is done using the below equation

PCTarget = (PCbeq + 4) + 4N	where N = immediate

Here, PCTarget = 0x00003060 and PCbeq = 0x00003040

So, 0x00003060 = (0x00003040 +4) + 4N

4N = 0x0000001c

N = 0x0x00000007 (Take only 16 bits) = 0007

The machine code is:

000100 01001 01011 0000 0000 0000 0111

0001 0001 0010 1011 0000 0000 0000 0111

So, the machine code in hexa decimal format is 0x112b0007

- Explanation with respect to the fourth line

i.e., multu $a0, $a0

This is an Mult R-type instruction, and the machine code is written as:

6 bits	           5 bits	           5 bits	            5 bits	            5 bits	            6 bits

| Opcode | Rs | Rt | Rd | Shamt | Function |
| --- | --- | --- | --- | --- | --- |

32 bits

Opcode: 000000-011001

Rs = $a0 = $4 = 00100

Rt = $a0 = $4 = 00100

Rd = 00000

Shamt = 00000

So, the machine code for the above is: 000000 00100 00100 00000 00000 011001

It can be written as: 0000 0000 1000 0100 0000 0000 0001 1001

Finally, it can be written as: 0x00840019

COLLABORATION SECTION:

- Written MIPS assembly program for the C++ algorithm of the given two tasks in the MARS by collaborating with each other.

- Assembled and executed the codes and observed all the operations and values.

- Collaborated with each other to understand how the value gets stored in lo and hi register upon doing multiplication and division operations.

- By collaborating with each other, debugged each line to know the contents of the relevant registers and recorded the memory values at certain addresses.

- Understanding of how the machine code comes for j-type instruction was tough, but by collaborating with each other, it was understandable.

CONCLUSION:

In conclusion, by assembling, simulating, and analyzing the converted MIPS programs gained familiarity with the ISA control structures and the hi and lo registers.