ABOUT THE AUTHOR

Harish Marepalli had done bachelor’s in Electronics and Communication Engineering at Gokaraju Rangaraju Institute of Engineering and Technology, Hyderabad, Telangana, India. He has 3 years of experience as a Systems Engineer in Tata Consultancy Services and his role was software developer. Currently, he is pursuing his master’s in Computer Engineering at San Jose State University. His native place is Hyderabad and mother tongue is Telugu. Additionally, he does speak English and Hindi and would love to connect with the people of any region and build network.

LIST OF CONTENTS

- INTRODUCTION

- MY GROUP

- GIVEN TASK

- STEPS TAKEN TO COMPLETE THE TASK

- TEST LOG

- SCREEN CAPTURES

- STACK STATUS DIAGRAM

- DISCUSSION SECTION

- COLLABORATION SECTION

- CONCLUSION

- APPENDIX

- INTRODUCTION

This activity is used to write a MIPS assembly code to build a 50-entry array with the base address 0x100. The array needs to be accessed to perform some arithmetic calculations, and the result of the calculation will be used as the input argument to the MIPS assembly program for the factorial function.

- MY GROUP

Name: Student_Team 6

Harish Marepalli – 016707314

Tirumala Saiteja Goruganthu – 016707210

- GIVEN TASK

- The task is to take the given assembly code and fill the missing code.

- The function of the program is to build a 50-entry array with the base address 0x100.

- To perform the required arithmetic calculations, an array needs to be accessed.

- Compute the factorial of a number using a recursive procedure.

- The result of the calculation will be used as the input argument to the MIPS assembly program for the factorial function.

- STEPS TAKEN TO COMPLETE THE TASK

- Understood the part of the given that is already provided.

- Imported the given lab4.asm file into the MARS software.

- Completed the actual missing part of the code under the lines “#your code goes in here.”

- Assigned the values to the registers as specified.

  - $a1 <- n

  - $a0 <- array base address

  - $s0 <- n!

- Implemented the factorial function as a recursive procedure.

- Written the final value of ‘n’ obtained from the arithmetic calculation to the memory location at address 0x00.

- The factorial of n is written to the memory location at address 0x10.

- Assembled the code and checked for any errors.

- Executed the code step by step.

- Observed, Understood, and Recorded the values in the given log table.

- TEST LOG

CMPE 200 Assignment 4 Test Log

Programmer’s Names:  HARISH MAREPALLI                        Date: 10/01/2022

Record the observed contents of registers and data memory after each instruction is executed.

| Addr | MIPS Instruction | Machine Code | Registers |  |  |  | Memory Content |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | $a1 | $sp | $ra | $v0 | [0x00] | [0x10] |
| 3034 | lw $t0, 100($a0) | 0x8c880064 | 0x00000032 | 0x00002ffc | 0x00000000 | 0x00000000 | 0x00000000 | 0x00000000 |
| 3038 | lw $t1, 120($a0) | 0x8c890078 | 0x00000032 | 0x00002ffc | 0x00000000 | 0x00000000 | 0x00000000 | 0x00000000 |
| 303c | add $t1, $t0, $t1 | 0x01094820 | 0x00000032 | 0x00002ffc | 0x00000000 | 0x00000000 | 0x00000000 | 0x00000000 |
| 3040 | addi $t0, $0, 30 | 0x2008001e | 0x00000032 | 0x00002ffc | 0x00000000 | 0x00000000 | 0x00000000 | 0x00000000 |
| 3044 | divu $t1, $t0 | 0x0128001b | 0x00000032 | 0x00002ffc | 0x00000000 | 0x00000000 | 0x00000000 | 0x00000000 |
| 3048 | mflo $a1 | 0x00002812 | 0x00000005 | 0x00002ffc | 0x00000000 | 0x00000000 | 0x00000000 | 0x00000000 |
| 304c | sw $a1, 0($0) | 0xac050000 | 0x00000005 | 0x00002ffc | 0x00000000 | 0x00000000 | 0x00000005 | 0x00000000 |
| 3050 | jal factorial | 0x0c000c19 | 0x00000005 | 0x00002ffc | 0x00003054 | 0x00000000 | 0x00000005 | 0x00000000 |
| 3054 | add $s0, $v0, $0 | 0x00408020 | 0x00000005 | 0x00002ffc | 0x00003054 | 0x00000078 | 0x00000005 | 0x00000000 |
| 3058 | sw $s0, 16($0) | 0xac100010 | 0x00000005 | 0x00002ffc | 0x00003054 | 0x00000078 | 0x00000005 | 0x00000078 |
| 305c | li $v0, 10 | 0x2402000a | 0x00000005 | 0x00002ffc | 0x00003054 | 0x0000000a | 0x00000005 | 0x00000000 |
| 3060 | syscall | 0x0000000c | 0x00000005 | 0x00002ffc | 0x00003054 | 0x0000000a | 0x00000005 | 0x00000000 |
| 3064 | addi $sp, $sp, -8 | 0x23bdfff8 | 0x00000001 | 0x00002fd4 | 0x00003090 | 0x00000000 | 0x00000005 | 0x00000000 |
| 3068 | sw $a1, 4($sp) | 0xafa50004 | 0x00000001 | 0x00002fd4 | 0x00003090 | 0x00000000 | 0x00000005 | 0x00000000 |
| 306c | sw $ra, 0($sp) | 0xafbf0000 | 0x00000001 | 0x00002fd4 | 0x00003090 | 0x00000000 | 0x00000005 | 0x00000000 |
| 3070 | addi $t0, $0, 2 | 0x20080002 | 0x00000001 | 0x00002fd4 | 0x00003090 | 0x00000000 | 0x00000005 | 0x00000000 |
| 3074 | slt $t0, $a1, $t0 | 0x00a8402a | 0x00000001 | 0x00002fd4 | 0x00003090 | 0x00000000 | 0x00000005 | 0x00000000 |
| 3078 | beq $t0, $0, else | 0x11000003 | 0x00000001 | 0x00002fd4 | 0x00003090 | 0x00000000 | 0x00000005 | 0x00000000 |
| 307c | addi $v0, $0, 1 | 0x20020001 | 0x00000001 | 0x00002fd4 | 0x00003090 | 0x00000001 | 0x00000005 | 0x00000000 |
| 3080 | addi $sp, $sp, 8 | 0x23bd0008 | 0x00000001 | 0x00002fdc | 0x00003090 | 0x00000001 | 0x00000005 | 0x00000000 |
| 3084 | jr $ra | 0x03e00008 | 0x00000001 | 0x00002fdc | 0x00003090 | 0x00000001 | 0x00000005 | 0x00000000 |
| 3088 | addi $a1, $a1, -1 | 0x20a5ffff | 0x00000001 | 0x00002fdc | 0x00003090 | 0x00000000 | 0x00000005 | 0x00000000 |
| 308c | jal factorial | 0x0c000c19 | 0x00000001 | 0x00002fdc | 0x00003090 | 0x00000000 | 0x00000005 | 0x00000000 |
| 3090 | lw $ra, 0($sp) | 0x8fbf0000 | 0x00000004 | 0x00002ff4 | 0x00003054 | 0x00000018 | 0x00000005 | 0x00000000 |
| 3094 | lw $a1, 4($sp) | 0x8fa50004 | 0x00000005 | 0x00002ff4 | 0x00003054 | 0x00000018 | 0x00000005 | 0x00000000 |
| 3098 | addi $sp, $sp, 8 | 0x23bd0008 | 0x00000005 | 0x00002ffc | 0x00003054 | 0x00000018 | 0x00000005 | 0x00000000 |
| 309c | mul $v0, $a1, $v0 | 0x70a21002 | 0x00000005 | 0x00002ffc | 0x00003054 | 0x00000078 | 0x00000005 | 0x00000000 |
| 30a0 | jr $ra | 0x03e00008 | 0x00000005 | 0x00002ffc | 0x00003054 | 0x00000078 | 0x00000005 | 0x00000000 |
| 30a4 |  |  |  |  |  |  |  |  |

- SCREEN CAPTURES

  - Figures 1 and 2 are screenshots of the assembly code in the MARS editor.

Fig. 1

Fig. 2

  - Figure 3 is the screenshot of the execution window after assembling and before executing the code in MARS.

Fig. 3

  - Figure 4 is the screenshot of the execution window after assembling and after executing the code in MARS.

Fig. 4

- STACK STATUS DIAGRAM

Figures 5 and 6 in the appendix are the stack status diagrams that show the addresses, stack pointer position, and values of $a1 and $ra after each iteration.

- When n=5, $a1 = 0x00000005 and $ra = 0x00003054

- When n=4, $a1 = 0x00000004 and $ra = 0x00003090

- When n=3, $a1 = 0x00000003 and $ra = 0x00003090

- When n=2, $a1 = 0x00000002 and $ra = 0x00003090

- When n=1, $a1 = 0x00000001 and $ra = 0x00003090

- When n=0: In this case also 2 locations will be allocated. Since the condition fails at this iteration, these 2 locations are deallocated again.

- DISCUSSION SECTION

Following are the observations related to stack status:

- For every iteration, the value in the Stack Pointer register changes.

- The old values remain in the Stack Pointer register and the values get stacked up.

- The value of a1 register changes accordingly with the value for which the factorial has to be calculated.

- The value of the return address also can be noted from the stack.

- At last, when the value of n reduces to zero, two more locations will be allocated and deallocated automatically.

- This process is a recursive procedure since there is a recursive call in the program.

- The values in the array get stored in the data segment addresses.

A sample stack diagram is shown below. The actual stack status diagram is shown in the appendix.

0x00000005

0x00003054

The top cell’s code is 0x00002ffc and it is also called SPold.

The cell which has the value 0x00000005 is a1 and it’s code is 0x00002ff8.

The cell which has the value 0x00003054 is return address and it is also the SPnew. It’s address is 0x00002ff4.

- COLLABORATION SECTION

- Understood the array operations by having a discussion and collaboration.

- Understood how the arithmetic calculation is done in the MIPS program.

- Written MIPS assembly program for the factorial program in MARS by collaborating with each other.

- Understood how to perform a recursive factorial operation.

- Discussed about the stack status and understood how the stack pointer changes for every iteration and how the values get stacked up by collaborating with each other.

- Assembled and executed the codes and observed all the operations and values.

- By collaborating with each other, debugged each line to know the contents of the relevant registers and recorded the memory values at certain addresses.

- Understanding the stack operation was something was challenging, but by collaborating with each other, it was understandable.

- CONCLUSION

In conclusion, by writing, assembling, and executing, gained familiarity with the MIPS implementation of arrays, stacks, procedures, and recursive procedures. I also understood the MIPS ISA, assembly programming, as well as testing. Finally, from this assignment, I gained familiarity about how the stack works.

APPENDIX

SOURCE CODE:

# $a0 = array base address

# $a1 = n

# $s0 = n!

Main:

addi $a0, $0, 0x100     	# array base address = 0x100

addi $a1, $0, 0        		# i = 0

addi $t0, $0, 3

addi $t1, $0, 50        	# $t1 = 50

CreateArray_Loop:

slt $t2, $a1, $t1       		# i < 50?

beq $t2, $0, Exit_Loop  	# if not then exit loop

sll $t2, $a1, 2         		# $t2 = i * 4 (byte offset)

add $t2, $t2, $a0       	# address of array[i]

mult $a1, $t0

mflo $t3                		# $t3 = i * 3

sw $t3, 0($t2)          	# save array[i]

addi $a1, $a1, 1        	# i = i + 1

j CreateArray_Loop

Exit_Loop:

# your code goes in here...

# arithmetic calculation

lw $t0, 100($a0)	    	# loading the content of register t0 at location 100+$a0 - my_array[25]

lw $t1, 120($a0)	    	# loading the content of register t1 at location 120+$a0 - my_array[30]

add $t1, $t0, $t1	    	# my_array[25] + my_array[30]

addi $t0, $0, 30	    	# assigned the value 30 to the register t0

divu $t1, $t0	    	# (my_array[25] + my_array[30])/30

mflo $a1		    	# moving the li content to the register a1

sw $a1, 0($0)	    	# n = (my_array[25] + my_array[30])/30. n = $a1

# factorial computation

jal factorial           		# call procedure

add $s0, $v0, $0        	# return value

sw $s0, 16($0)	    	# storing the s0 register content at a location 0x10

li $v0, 10		    	# system call for exit

syscall		    	# Exit!

factorial:

addi $sp, $sp, -8       	# make room on the stack

sw $a1, 4($sp)          	# store $a1

sw $ra, 0($sp)          	# store $ra

# your code goes in here

addi $t0, $0, 2	    	# $t0 = 2

slt $t0, $a1, $t0	    	# n<=1?

beq $t0, $0, else	    	# no: go to else (recursion)

addi $v0, $0, 1	    	# yes: return 1

addi $sp, $sp, 8	    	# restore $sp

jr $ra		    	# return

else:

addi $a1, $a1, -1	    	# n = n-1

jal factorial	    		# recursive call

lw $ra, 0($sp)	    	# restore return address

lw $a1, 4($sp)	    	# restore input

addi $sp, $sp, 8	    	# restore $sp

mul $v0, $a1, $v0	    	# n*factorial(n-1)

jr $ra		    	# return

This is the source code for running the factorial. It involves several MIPS instructions.

STACK STATUS DIAGRAM:

Fig. 5

Fig. 6